- Chuẩn bị

  - Power

Cắm pin Hoverboard, pin các Cảm biến, pin Màn hình, pin Jetson.

Bật tất cả công tắc (màu đỏ) của các pin.

  - Ngoại vi

Đảm bảo tất các sensors và màn hình được kết nối với Jetson.

Bao gồm:

- LiDAR (LAN)

- IMU (I2C)

- Hoverboard (USB-A) thông qua module CP2102 - Mạch Chuyển Đổi USB To TTL UART

  - Nhấn button (bên hông, cạnh công tắc) để khởi động Hoverboard.

⇒ Kêu bíp bíp nghĩa là đã bật, chỉ khi di chuyển mới hết kêu.

- Ultrasonic (USB-A) - hiện tại chưa dùng được.

- Màn hình:

  - Cảm ứng (USB-A)

  - HDMI

- USB Wifi (USB-A): chỉ dùng cho chức năng chuyển file từ thiết bị khác sang Jetson (phải chung wifi với Jetson).

- Pre-check

Cách 1: dùng UI

Nhấn pre_check trên desktop

Cách 2: dùng command

cd mbrobot_ws

./pre_check.sh

Lưu ý:

- Nếu chưa ping được LiDAR thì thực hiện lại cho đến khi được (3-4 lần) vì pha khởi động của LiDAR … hoặc dùng lệnh:

sudo ip addr add 192.168.11.1/24 dev enp2s0

sudo ip link set enp2s0 up

- Run

Sau khi đảm bảo đã thực hiện các bước trên:

- Bước 1: Đặt robot đúng với Initial Pose đã xác định (Trước Thang máy và quay lưng với phía Thang máy).

Hình 1: Initial Pose

- Bước 2: Nhấn start_nav.sh trên desktop, nếu không báo lỗi gì và hiện ra dòng bên dưới là OK.

- Bước 3: Nhấn … trên desktop để mở App và sử dụng.

Nếu robot chạy không ổn định hoặc sai thì dừng chương trình của 2 bước trên rồi chạy lại.

# Troubleshooting

Per-subsystem bring-up checks. Run each one in isolation before blaming the full stack — the launch files start ten processes and a single silent device makes the failure look like a Nav2 problem.

Commands assume the workspace is at ~/mbrobot_ws and has been sourced:

source ~/mbrobot_ws/install/setup.bash

## Pre-flight check

pre_check.sh runs six phases: build, system prerequisites, hardware and devices, ROS topics and TF, configuration files for the selected floor, and disk / memory / temperature.

./pre_check.sh --help

./pre_check.sh --floor e6                       # full check, floor e6

./pre_check.sh --floor e1 --skip-build          # skip the colcon build phase

./pre_check.sh --floor e6 --check-topics        # also launch nodes and measure topic rates

Phase 0 rebuilds the workspace and phase 3 with --check-topics launches nodes in the background, so this is not a read-only script.

## LiDAR link

The RPLIDAR S2E is reached over Ethernet/UDP. The Jetson interface needs a static address on the sensor's subnet before sllidar_node can connect. Replace the interface name with whichever one is wired to the sensor:

sudo ip addr add 192.168.11.1/24 dev enp2s0

sudo ip link set enp2s0 up

Then check the scan is flowing:

ros2 topic hz /scan

ros2 topic hz /scan_filtered

If /scan is alive but /scan_filtered is not, the filter chain is failing its TF lookup to base_footprint rather than the LiDAR being at fault.

## Ultrasonic bridge

Run the bridge directly to see the raw frames arriving from the ESP32:

python3 src/mobile_robot/scripts/jetson_sensor_bridge.py --debug

A frame is accepted only if it starts with $, contains *, carries exactly six fields, and matches its XOR checksum. Silent output with no errors means the port opened but nothing is being received — check /dev/ttyUltrasonic and the ESP32.

ros2 topic echo /ultrasonic/us_top_left

ros2 topic hz /ultrasonic_scan

## IMU

ros2 run bno055 bno055 \

--ros-args \

--params-file src/mobile_robot/config/bno055_params.yaml \

--remap /imu/imu:=/imu/data

In a second terminal:

ros2 topic list | grep imu     # expect /imu/data /imu/mag /imu/temp /imu/calib_status

ros2 topic echo /imu/data

ros2 topic hz /imu/data

The remap matters: without it the driver publishes on /imu/imu and the EKF subscribes to nothing.

## Wheel odometry

ros2 run mobile_robot wheel_odom_node \

--ros-args --params-file src/mobile_robot/config/wheel_odom_params.yaml

ros2 topic hz /odom

ros2 topic echo /odom

The node opens /dev/ttyWheel once at startup and does not reconnect, so a serial error leaves the node alive but permanently silent. Restart it rather than waiting.

## EKF

ros2 run robot_localization ekf_node \

--ros-args --params-file src/mobile_robot/config/ekf.yaml

ros2 topic echo /odometry/filtered

The EKF is the only publisher of TF odom -> base_footprint. If that transform is missing, check that wheel_odom_node has publish_tf: false and that both /odom and /imu/data are live — the filter will not publish until it has an input.

## Inspecting the running graph

ros2 node list

ros2 node list | grep server      # Nav2 lifecycle servers only

rqt_graph

ros2 param get /bt_navigator default_bt_xml_filename

ros2 run tf2_tools view_frames

## Common ordering symptom

canTransform warnings in the first seconds of a run are expected: Nav2 activates at t=12 s while global_localizer only starts at t=14 s, so there is a short window with no map -> odom transform. Warnings that persist past roughly t=20 s are a real localization failure, not a start-up artefact.

# Mapping

slam.launch.py brings up the hardware, the EKF and slam_toolbox in online_async mode. It starts no teleop node, so drive the robot manually:

ros2 run teleop_twist_keyboard teleop_twist_keyboard

run_slam.sh wraps the launch file with bag recording and an automatic map save on exit:

./run_slam.sh --scenario S6 --speed 0.10 --run 01

./run_slam.sh --no-save-map --no-bag        # dry run

./run_slam.sh --help

## Saving a map manually

Confirm the map topic is alive first:

ros2 topic hz /map

### Preferred: raise the timeout and match the QoS

slam_toolbox publishes /map with transient-local durability, and large maps exceed the map saver's default timeout. Set both:

ros2 run nav2_map_server map_saver_cli \

-f ~/mbrobot_ws/src/mobile_robot/maps/map_e6 \

--ros-args \

-p save_map_timeout:=60000 \

-p map_subscribe_transient_local:=true \

-p free_thresh_default:=0.25 \

-p occupied_thresh_default:=0.65

### Alternative: the slam_toolbox service

ros2 service call /slam_toolbox/save_map slam_toolbox/srv/SaveMap \

"name: {data: '$HOME/mbrobot_ws/src/mobile_robot/maps/map_e6'}"

## After saving

map_saver_cli writes a .pgm and a .yaml. Keep the image: key in the .yaml relative to the file itself so the map stays portable across machines:

image: map_e6.pgm

The floor launch argument selects maps/map_<floor>.yaml together with config/checkpoints_<floor>.yaml, so a new map needs a matching checkpoint file before navigator can use it.

scripts/tools/save_checkpoints.py captures poses from /amcl_pose on each ENTER keypress and writes them as JSON. navigator reads the YAML schema shown in the existing config/checkpoints_*.yaml files, so the captured output has to be converted before use.

Re-run colcon build after adding maps or checkpoints — the launch files read them from install/share/, not from the source tree.